# Function Flow

# Block Diagram:

The system is provided with 2 keys and 3 LEDS

- Keys

  - Selection keys

  - Navigation keys

- LED

  - Green- Motor LED- motor run indication

  - Amber-Pause LED- mode pause indication

  - Red- Fault LED – Fault indication

# INITIAL SCREEN:

Initially: The system retrieves the previous mode and check for the home position Where the screen will display number of cycle from previous/Default mode

Action:

On pressing the key1 the display changes to “SNG LNE” mode, here if the key2 is pressed

user will navigate through different mode, if key 1 is pressed on selecting the required mode, then the user will enter that particular mode. If no key is in action,  it will return back to the home screen.

# SINGLE LINE MODE:

## Entering Single Line Mode:

    - On pressing the key 1 will make the user enters the “SGN LINE” mode.

## Functionality with in Single Line Mode:

    - When the system is in SGN LINE mode, pressing Key 1 changes the display to "CYCLE-00".

    - The user can then set the desired number of cycles.

    - After 2 second of confirming the cycle count, the system automatically enters the Pause Time setting mode.

    - After 2 seconds of setting the Pause Time, the system automatically saves this setting and enters the RUN time (after sensing the Home sensor).

## Function in each Mode:

- Cycles-

In this mode, the user can set the number of cycles using increment (key 1) and decrement (key 2) keys, if no key is in action for 2 second the screen changes to “PASETIME” then after 2 seconds, screen changes to “CYCLE-00” (home screen/Run time).

- Pause time-

In this mode the user can set the pause time using increment (key 1) and decrement (key 2) keys if no key is in action for 2 second the screen changes to “CYCLE-00” (home screen/Run time).

# PROGRESSIVE MODE:

## Entering Progressive Mode:

    - On pressing the key 1 will make the user enters the “PROGSIVE” mode.

## Functionality with in Progressive Mode:

    - When the system is in PROGRESSIVE mode, pressing Key 1 changes the display to "CYCLE-00".

    - The user can then set the desired number of cycles.

    - After 2 second of confirming the cycle count, the system automatically enters the Pause Time setting mode.

    - After 2 seconds of setting the Pause Time, the system automatically saves this value and enters the RUN time.

## Function in each Mode:

- Cycles-

In this mode, the user can set the number of cycles using increment (key 1) and decrement (key 2) keys, if no key is in action for 2 second the screen changes to “PROGSIVE” then to “CYCLE-00” (home screen).

- Pause time-

In this mode the user can set the pause time using increment (key 1) and decrement (key 2) keys if no key is in action for 2 second the screen changes to “PROGSIVE” then to “CYCLE-00” (home screen)

# WORK IN EACH MODE:

- SINGLE LINE MODE: INPUTS:

  - CYCLE: The user specifies how many cycles the system should complete

(maximum cycle:50).

  - PAUSE TIME: The user sets the duration for which the system will pause after completing given number of cycles

# CYCLE EXECUTION:

  - The cycle begins from the Home Position.

  - The motor starts moving forward.

  - Upon sensing the Stroke Sensor, the motor reverses direction and moves backward until it senses the Home Sensor.

  - This cycle repeats for (N - 1) cycles, following the same forward and reverse pattern.

  - In the Nᵗʰ cycle:

    - The motor moves forward.

    - When the Relief Sensor is sensed, the system enters Pause Time.

# PROGRESSIVE MODE: INPUTS:

  - CYCLE: The user specifies how many cycles the system should complete.

  - PAUSE TIME: The user sets the duration for which the system will pause after completing given number of cycles

# CYCLE EXECUTION:

  - The cycle begins from the Home Position.

  - The motor starts moving forward.

  - Upon sensing the Stroke Sensor, the motor reverses direction and moves backward until it senses the Home Sensor.

  - This cycle repeats for N cycles, following the same forward and reverse pattern. Then enters the pause time

LED INDICATION:

| LED COLOUR | Description |
| --- | --- |
| GREEN LED | MOTOR RUN Indication |
| AMBER LED | MOTOR PAUSE Indication |
| RED LED | FAULT Indication |