# Function Flow:

The system is provided with 2 keys and 3 LEDS

- Keys

  - Selection keys

  - Navigation keys

- LED

  - Green- Motor LED- motor run indication

  - Amber-Pause LED- mode pause indication

  - Red- Fault LED – Fault indication

# INITIAL SCREEN:

Initially: The system retrieves the Default mode and check for the home position Where the screen will display number of cycles from Default mode.

Key operation:

- Pressing Key1 navigates the user to the Factory Settings menu.

- A short press of Key2 displays the current setting.

- A long press of Key2 allows the user to modify the selected setting.

- Pressing Key1 and Key2 simultaneously initiates the Password Reset process.

# Factory setting:

Seven modes:

- Pressing Key1 cycles sequentially through all seven available operating modes displayed on the screen.

- Pressing Key1 and Key2 simultaneously selects the currently displayed mode as the primary mode and allows the user to select a second mode, thereby activating Dual Mode operation.
Dual Mode is supported only for Single Point Stand-Alone and Multi Point Stand-Alone modes. For all other modes, the Key1 + Key2 action has no effect and Dual Mode will not be enabled.

- Pressing Key2 selects the currently displayed mode and prompts the user to either create a new password or enter an existing password, depending on the current system state.

# Password Creation, Reset Procedure and Entry:

1. Password Creation

- When password creation is initiated, the display shows “PW”.

- The user enters the desired password using number entry (0–9).

- After successful entry, the system stores the password and displays “PW SAVED”.

- Once the password is saved, the system automatically returns to the Home Screen (e.g., “CYCLE-00”).

2. Password Reset

- When the Password Reset function is initiated, the display shows “PW RESET”.

- The system prompts the user to enter the existing password, displayed as “OLD PW”.

- If the entered old password is valid, the system displays “OK” to confirm verification and proceeds to prompt the user to enter a new password (displayed as “NEW PW”).

- If the entered old password is invalid, the system displays “WRONG” and returns to the previous operation without allowing password reset.

- Upon successful entry and storage of the new password, the system updates the password and returns to the Home Screen.

3. Password Entry

- When password entry is required, the display shows masked characters (e.g., “****”).

- Key1 is used for numeric entry (0–9).

- Key2 is used for digit/placement selection within the password.

- After the complete password is entered, the system validates the password:

  - If the password is correct, access is granted and the system proceeds to the requested operation.

  - If the password is incorrect, the user is prompted to re-enter the password.

# User setting:

Mode Selection (Dual Mode Only)

- “MP-SA”: Represents the Multi Point Stand-Alone mode.

- “SP-SA”: Represents the Single Point Stand-Alone mode.

- Key Interaction:

  - Key1: Navigates between the two modes (MP-SA ↔ SP-SA).

  - Key2: Selects the currently displayed mode and enters the cycle setting.

Cycle Setting

- Key1 (Increment): Increases the cycle count.

- Key2 (Decrement): Decreases the cycle count.

- Timeout / No Key Action: If no key is pressed for 2 seconds, the system automatically moves to the Process Time (PT) setting.

Process Time (PT) Parameter

- Key1 (Increment): Increases the Pause Time value.

- Key2 (Decrement): Decreases the Pause Time value.

- Timeout / No Key Action: If no key is pressed for 2 seconds, the system exits the settings menu and returns to the Home Screen.